The Wonderful Wizardry

of

Finding a Gene

The paraphernalia you will need

| Gel Spell Green chromosome samples Micropipettor Micropipette tips Black card | Carbon fibre electrodes Set of +/- electrical wires 3 x 9v batteries Discard jar |
| --- | --- |
| Electrophoresis tank containing agar gel covered with water |  |

- Place 0.6 g agar powder and 20 cm3 water into a 100 cm3 beaker.

- Heat over a hotplate until the agar goes clear, swirling occasionally to prevent the agar burning.

- Remove beaker from heat and place on a heatproof mat to cool (this will also allow time for most of the air bubbles in the agar to disperse).

- Fit the comb inside the electrophoresis tank and place on a level surface.

- Once the agar has cooled enough to hold, pour the molten agar into the central cavity of the tank ensuring that it flows through the teeth of the comb.

- Leave the gel to set then cover with water and remove the comb.

### Electrophoresis Spell

- Using the micropipettors and tips load the first well in the gel with 20 l of the sample labelled F.  Place the black card under the tank to help see the wells.

- Repeat the first step using each green sample in turn till you have loaded:

| well 1 - F | well 2 - W | well 3 - C | well 4 - P |
| --- | --- | --- | --- |

- Insert the carbon fibre electrodes at each end of the tank and attach the crocodile clips to them.  The black (-ve) clips go at the end nearest the wells.

- Wire up the electrophoresis tank to the batteries, with the red clip at the vacant +ve terminal of the battery.

- Leave for 30 minutes and then remove wires from battery and tank.

- Pour off the water and examine the gel.